// top/core_top_sim.v (完整修复版 - 移除 interrupt_handler 相关代码)

`timescale 1ns/1ps

module core_top_sim (

input  wire        clk_i,

input  wire        rst_n_i,

output wire        uart_tx_o,

// ========== 调试输出端口 ==========

output wire [31:0] debug_if_pc,

output wire [31:0] debug_if_instr,

output wire [31:0] debug_id_rs1_data,

output wire [31:0] debug_id_rs2_data,

output wire [31:0] debug_ex_alu_result,

output wire [31:0] debug_ex_mem_addr,

output wire [31:0] debug_ex_mem_wdata,

output wire        debug_ex_mem_we,

output wire [2:0]  debug_ex_mem_width,

output wire        debug_mem_bus_we,

output wire [31:0] debug_mem_bus_addr,

output wire [31:0] debug_mem_bus_wdata,

output wire [31:0] debug_mem_bus_rdata,

output wire        debug_bus_uart_we,

output wire [31:0] debug_bus_uart_addr,

output wire [31:0] debug_bus_uart_wdata,

output wire [4:0]  debug_id_rs1_addr,

output wire [4:0]  debug_id_rs2_addr,

//寄存器

output wire [31:0] debug_x0,

output wire [31:0] debug_x1,

output wire [31:0] debug_x2,

output wire [31:0] debug_x3,

output wire [31:0] debug_x4,

output wire [31:0] debug_x5_t0,

output wire [31:0] debug_x6_t1,

output wire [31:0] debug_x7_t2,

output wire [31:0] debug_x8_t3,

output wire [31:0] debug_x9_t4,

output wire [31:0] debug_x10_a0,

output wire [31:0] debug_x11_a1,

output wire [31:0] debug_x12_a2,

output wire [31:0] debug_x13,

output wire [31:0] debug_x14,

output wire [2:0]  debug_uart_state,

output wire [31:0] debug_uart_baud_cnt,

output wire [3:0]  debug_uart_bit_cnt,

output wire [7:0]  debug_uart_shift_reg,

output wire [7:0]  debug_uart_fifo_data0,

output wire [7:0]  debug_uart_fifo_data1,

output wire [7:0]  debug_uart_fifo_data2,

output wire [7:0]  debug_uart_fifo_data3,

output wire [7:0]  debug_uart_fifo_data4,

output wire [7:0]  debug_uart_fifo_data5,

output wire [7:0]  debug_uart_fifo_data6,

output wire [7:0]  debug_uart_fifo_data7,

output wire [7:0]  debug_uart_fifo_data8,

output wire [7:0]  debug_uart_fifo_data9,

output wire [7:0]  debug_uart_fifo_data10,

output wire [7:0]  debug_uart_fifo_data11,

output wire [7:0]  debug_uart_fifo_data12,

output wire [7:0]  debug_uart_fifo_data13,

output wire [7:0]  debug_uart_fifo_data14,

output wire [7:0]  debug_uart_fifo_data15,

output wire [3:0]  debug_uart_wr_ptr,

output wire [3:0]  debug_uart_rd_ptr,

output wire [4:0]  debug_uart_fifo_count,

output wire        debug_uart_fifo_full,

output wire        debug_uart_fifo_empty,

output wire        debug_uart_fifo_we,

output wire        debug_uart_fifo_re,

// CSR/中断调试输出

output wire [31:0] debug_mstatus,

output wire [31:0] debug_mie,

output wire [31:0] debug_mtvec,

output wire [31:0] debug_mepc,

output wire [31:0] debug_mcause,

output wire [31:0] debug_mip,

output wire        debug_interrupt_pending,

output wire        debug_interrupt_taken,

//中断调试

output wire [31:0] debug_csr_inst_pc,

output wire        debug_csr_inst_valid,

output wire [31:0] debug_csr_inst_instr,

output wire        debug_csr_write,

output wire [11:0] debug_csr_write_addr,

output wire [31:0] debug_csr_write_data,

output wire [31:0] debug_t0_value,

output wire [31:0] debug_instr_4,

output wire [31:0] debug_instr_5,

output wire [31:0] debug_instr_6,

output wire [31:0] debug_instr_7,

output wire [31:0] debug_ex_csr_result,

output wire [31:0] debug_ex_mem_csr_result,

//定时计数

output wire [6:0] debug_timer_counter,

output wire        debug_timer_irq,

// ID阶段

output wire        debug_id_csr_inst,

output wire [11:0] debug_id_csr_addr,

output wire [2:0]  debug_id_csr_op,

output wire [4:0]  debug_id_csr_zimm,

// ID/EX阶段

output wire        debug_ex_csr_inst,

output wire [11:0] debug_ex_csr_addr,

output wire [2:0]  debug_ex_csr_op,

output wire [4:0]  debug_ex_csr_zimm,

// CSR指令处理模块

output wire [2:0]  debug_csr_inst_op,

output wire [11:0] debug_csr_inst_addr,

output wire [4:0]  debug_csr_inst_rs1,

output wire [31:0] debug_csr_inst_rs1_data,

output wire [31:0] debug_csr_inst_imm,

output wire [31:0] debug_csr_inst_rdata,

output wire        debug_csr_inst_we,

output wire [11:0] debug_csr_inst_waddr,

output wire [31:0] debug_csr_inst_wdata,

output wire [31:0] debug_csr_inst_result,

// 最终CSR写信号

output wire        debug_final_csr_we,

output wire [11:0] debug_final_csr_waddr,

output wire [31:0] debug_final_csr_wdata,

output wire        debug_interrupt_taken_pipe,

output wire [1:0]  debug_interrupt_hold_cnt,

output wire        debug_interrupt_accepted,

output wire        debug_interrupt_condition,

output wire [4:0]  debug_interrupt_condition_bits,

output wire [31:0] debug_selected_pc,

output wire [2:0]  debug_selected_stage,

// CSR寄存器文件接口

output wire [31:0] debug_csr_reg_rdata,

output wire        debug_csr_reg_we,

output wire [11:0] debug_csr_reg_waddr,

output wire [31:0] debug_csr_reg_wdata,

//前递调试信号

output wire [1:0]  debug_forwardA,

output wire [1:0]  debug_forwardB,

output wire [4:0]  debug_id_ex_rs1,

output wire [4:0]  debug_id_ex_rs2,

output wire [4:0]  debug_ex_mem_rd,

output wire        debug_ex_mem_reg_we,

output wire        debug_ex_mem_mem_re,

output wire [4:0]  debug_mem_wb_rd,

output wire        debug_mem_wb_reg_we,

output wire [31:0] debug_ex_mem_alu_result,

output wire [31:0] debug_mem_forward_data,

output wire [31:0] debug_op1_selected,

output wire [31:0] debug_op2_selected,

output wire [4:0]  debug_rs1_addr_id,

output wire [4:0]  debug_rs2_addr_id,

output wire [31:0] debug_rs1_data_id,

output wire [31:0] debug_rs2_data_id,

output wire        debug_id_ex_flush,

output wire        debug_id_ex_intr_flush,

output wire        debug_id_ex_stall,

// Hazard Unit 调试信号

output wire        debug_stall_if,

output wire        debug_stall_id,

output wire        debug_flush_if,

output wire        debug_flush_id,

output wire        debug_load_use_hazard,

output wire        debug_control_hazard,

output wire [4:0]  debug_hazard_rs1_addr,

output wire [4:0]  debug_hazard_rs2_addr,

output wire [4:0]  debug_hazard_ex_rd_addr,

output wire        debug_hazard_ex_reg_we,

output wire        debug_hazard_ex_mem_re,

output wire [4:0]  debug_fwd_ex_mem_rd,

output wire        debug_fwd_ex_mem_reg_we,

output wire [4:0]  debug_fwd_mem_wb_rd,

output wire        debug_fwd_mem_wb_reg_we,

output wire [31:0] debug_ex_op1,

output wire [31:0] debug_ex_op2,

output wire [31:0] debug_ex_rs1_original,

output wire [31:0] debug_ex_rs2_original,

output wire [31:0] debug_next_pc,

output wire        debug_pc_changed,

// GPIO 调试信号

output wire [31:0] debug_gpio_out,

output wire [31:0] debug_gpio_oe,

output wire [31:0] debug_gpio_in,

output wire [31:0] debug_gpio_if,

output wire        debug_gpio_interrupt,

output wire        debug_bus_gpio_we,

output wire        debug_bus_gpio_re,

output wire [31:0] debug_bus_gpio_addr,

output wire [31:0] debug_bus_gpio_wdata,

output wire [31:0] debug_bus_gpio_rdata,

// Timer 调试信号

output wire [31:0] debug_timer_load,

output wire [31:0] debug_timer_count,

output wire        debug_timer_enable,

output wire        debug_timer_irq_flag,

output wire        debug_timer_interrupt

);

// ==========================================================================

// CSR地址常量定义

// ==========================================================================

localparam CSR_MSTATUS = 12'h300;

localparam CSR_MIE     = 12'h304;

localparam CSR_MTVEC   = 12'h305;

localparam CSR_MEPC    = 12'h341;

localparam CSR_MCAUSE  = 12'h342;

localparam CSR_MIP     = 12'h344;

// ==========================================================================

// 内部信号声明

// ==========================================================================

// GPIO外部引脚

wire [31:0] gpio_in_test;

wire [31:0] gpio_out_test;

wire [31:0] gpio_oe_test;

wire        gpio_interrupt;

wire [31:0] gpio_out_val;

wire [31:0] gpio_oe_val;

wire [31:0] gpio_in_val;

wire [31:0] gpio_if_val;

// 定时器中断

wire        timer_interrupt;

wire [31:0] timer_load_val;

wire [31:0] timer_count_val;

wire        timer_enable;

wire        timer_irq_flag;

// IFU

wire [31:0] if_pc;

wire [31:0] if_instr;

wire [31:0] pc_plus4;

// IF/ID

wire [31:0] if_id_pc;

wire [31:0] if_id_instr;

// ID

wire [31:0] id_rs1_data;

wire [31:0] id_rs2_data;

wire [31:0] id_imm;

wire [4:0]  id_rs1_addr;

wire [4:0]  id_rs2_addr;

wire [4:0]  id_rd_addr;

wire [3:0]  id_alu_op;

wire        id_alu_src;

wire        id_mem_we;

wire        id_mem_re;

wire [1:0]  id_wb_sel;

wire        id_reg_we;

wire        id_branch;

wire        id_jump;

wire [2:0]  id_funct3;

wire [2:0]  id_mem_width;

wire [6:0]  id_opcode;

wire        id_csr_inst;

wire [11:0] id_csr_addr;

wire [2:0]  id_csr_op;

wire [4:0]  id_csr_zimm;

// ID/EX

wire [31:0] id_ex_pc;

wire [31:0] id_ex_rs1_data;

wire [31:0] id_ex_rs2_data;

wire [31:0] id_ex_imm;

wire [4:0]  id_ex_rs1_addr;

wire [4:0]  id_ex_rs2_addr;

wire [4:0]  id_ex_rd_addr;

wire [3:0]  id_ex_alu_op;

wire        id_ex_alu_src;

wire        id_ex_mem_we;

wire        id_ex_mem_re;

wire [1:0]  id_ex_wb_sel;

wire        id_ex_reg_we;

wire        id_ex_branch;

wire        id_ex_jump;

wire [2:0]  id_ex_funct3;

wire [2:0]  id_ex_mem_width;

wire [6:0]  id_ex_opcode;

wire        id_ex_mret;

wire        id_ex_csr_inst;

wire [11:0] id_ex_csr_addr;

wire [2:0]  id_ex_csr_op;

wire [4:0]  id_ex_csr_zimm;

// EX

wire [31:0] ex_alu_result;

wire [31:0] ex_mem_addr;

wire [31:0] ex_mem_wdata;

wire        ex_mem_we;

wire        ex_mem_re;

wire [2:0]  ex_mem_width;

wire        ex_branch_taken;

wire        ex_jump_taken;

wire [31:0] ex_branch_target;

wire [31:0] ex_jump_target;

wire [31:0] ex_pc_plus4;

wire [31:0] ex_result;

wire [1:0]  ex_wb_sel;

wire        ex_reg_we;

wire [4:0]  ex_rd_addr;

wire [31:0] op1_selected;

wire [31:0] op2_selected;

wire [31:0] ex_csr_result;

wire [31:0] forward_mem_data;

// EX/MEM

wire [31:0] ex_mem_alu_result;

wire [31:0] ex_mem_mem_addr;

wire [31:0] ex_mem_mem_wdata;

wire        ex_mem_mem_we;

wire        ex_mem_mem_re;

wire [2:0]  ex_mem_mem_width;

wire [31:0] ex_mem_pc_plus4;

wire [4:0]  ex_mem_rd_addr;

wire [1:0]  ex_mem_wb_sel;

wire        ex_mem_reg_we;

wire [31:0] ex_mem_csr_result;

wire [4:0]  ex_mem_rd_addr_for_hazard;

wire        ex_mem_reg_we_for_hazard;

wire        ex_mem_mem_re_for_hazard;

// MEM

wire [31:0] mem_alu_result;

wire [31:0] mem_mem_rdata;

wire [31:0] mem_pc_plus4;

wire [4:0]  mem_rd_addr;

wire [1:0]  mem_wb_sel;

wire        mem_reg_we;

wire [31:0] mem_csr_result;

// MEM/WB

wire [31:0] wb_alu_result;

wire [31:0] wb_mem_rdata;

wire [31:0] wb_pc_plus4;

wire [4:0]  wb_rd_addr;

wire [1:0]  wb_wb_sel;

wire        wb_reg_we;

wire [31:0] wb_csr_result;

wire        mem_wb_mem_re;

// WB

wire [31:0] wb_data;

wire        wb_reg_we_out;

wire [4:0]  wb_rd_addr_out;

// MEM Bus Interface

wire        mem_bus_re;

wire        mem_bus_we;

wire [31:0] mem_bus_addr;

wire [31:0] mem_bus_wdata;

wire [2:0]  mem_bus_width;

wire [31:0] mem_bus_rdata;

wire        mem_bus_ready;

// Bus Signals

wire        bus_ram_re;

wire        bus_ram_we;

wire [31:0] bus_ram_addr;

wire [31:0] bus_ram_wdata;

wire [2:0]  bus_ram_width;

wire [31:0] bus_ram_rdata;

wire        bus_ram_ready;

wire        bus_uart_we;

wire        bus_uart_re;

wire [31:0] bus_uart_addr;

wire [31:0] bus_uart_wdata;

wire [31:0] bus_uart_rdata;

wire        bus_gpio_we;

wire        bus_gpio_re;

wire [31:0] bus_gpio_addr;

wire [31:0] bus_gpio_wdata;

wire [31:0] bus_gpio_rdata;

wire        bus_timer_we;

wire        bus_timer_re;

wire [31:0] bus_timer_addr;

wire [31:0] bus_timer_wdata;

wire [31:0] bus_timer_rdata;

wire [7:0] tx_data;

wire       tx_valid;

wire [15:0] debug_fifo_out_data_o;

wire       direct_transfer_o;

wire [7:0] data_to_send_o;

wire       tx_ready;

wire       debug_fifo_we_reg;

// Hazard & Forwarding

wire [1:0] forwardA;

wire [1:0] forwardB;

wire       stall_if;

wire       stall_id;

wire       flush_if;

wire       flush_id;

wire       stall_ex;

wire       stall_mem;

wire       stall_wb;

wire       flush_ex;

wire       flush_mem;

wire       flush_wb;

wire        hazard_load_use;

wire        hazard_control;

wire [4:0]  hazard_rs1_addr;

wire [4:0]  hazard_rs2_addr;

wire [4:0]  hazard_ex_rd_addr;

wire        hazard_ex_reg_we;

wire        hazard_ex_mem_re;

wire [4:0]  fwd_ex_mem_rd;

wire        fwd_ex_mem_reg_we;

wire [4:0]  fwd_mem_wb_rd;

wire        fwd_mem_wb_reg_we;

// ==========================================================================

// 中断相关信号

// ==========================================================================

wire       intr_software = 1'b0;

wire       intr_timer;

wire       intr_external = gpio_interrupt;

wire [31:0] csr_rdata;

wire [31:0] mtvec;

wire [31:0] mepc;

wire [31:0] mcause;

wire [31:0] mie;

wire [31:0] mstatus;

wire [31:0] mip;

wire        id_mret;

wire        csr_inst_valid;

wire [2:0]  csr_op;

wire [11:0] csr_addr;

wire [4:0]  csr_rs1_addr;

wire [31:0] csr_rs1_data;

wire [31:0] csr_imm;

wire [31:0] csr_inst_result;

wire        csr_inst_we;

wire [11:0] csr_inst_waddr;

wire [31:0] csr_inst_wdata;

wire        csr_we;

wire [11:0] csr_waddr;

wire [31:0] csr_wdata;

wire [31:0] csr_result;

wire        intr_pending;

wire [31:0] intr_cause;

wire [31:0] intr_handler_addr;

wire        pipe_csr_mepc_we;

wire [31:0] pipe_csr_mepc_data;

wire        pipe_csr_mcause_we;

wire [31:0] pipe_csr_mcause_data;

wire        pipe_csr_mstatus_we;

wire [31:0] pipe_csr_mstatus_data;

wire        interrupt_taken_pipe;

wire        interrupt_flush_pipe;

wire [31:0] interrupt_pc_pipe;

wire        intr_flush_if;

wire        intr_flush_id;

wire        intr_flush_ex;

wire        intr_flush_mem;

wire        intr_flush_wb;

// ==========================================================================

// 实例化所有模块

// ==========================================================================

data_ram u_data_ram (

.clk_i     (clk_i),

.rst_n_i   (rst_n_i),

.we_i      (bus_ram_we),

.re_i      (bus_ram_re),

.width_i   (bus_ram_width),

.addr_i    (bus_ram_addr),

.wdata_i   (bus_ram_wdata),

.rdata_o   (bus_ram_rdata),

.ready_o   (bus_ram_ready)

);

assign stall_ex  = 1'b0;

assign stall_mem = 1'b0;

assign stall_wb  = 1'b0;

assign flush_ex  = ex_branch_taken || ex_jump_taken;

assign flush_mem = 1'b0;

assign flush_wb  = 1'b0;

ifu_top u_ifu_top (

.clk              (clk_i),

.rst_n            (rst_n_i),

.stall_i          (stall_if || intr_flush_if),

.branch_taken_i   (ex_branch_taken),

.jump_taken_i     (ex_jump_taken),

.branch_target_i  (ex_branch_target),

.jump_target_i    (ex_jump_target),

.interrupt_pending_i(interrupt_taken_pipe),

.mtvec_i           (mtvec),

.instr            (if_instr),

.pc               (if_pc),

.pc_plus4         (pc_plus4)

);

if_id_reg u_if_id_reg (

.clk_i           (clk_i),

.rst_n_i         (rst_n_i),

.stall_i         (stall_id),

.flush_i         (flush_id),

.intr_flush_i    (intr_flush_id),

.if_pc_i         (if_pc),

.if_instr_i      (if_instr),

.id_pc_o         (if_id_pc),

.id_instr_o      (if_id_instr)

);

id_top u_id_top (

.clk           (clk_i),

.rst_n         (rst_n_i),

.instr         (if_id_instr),

.pc            (if_id_pc),

.wb_we_i       (wb_reg_we_out),

.wb_rd_addr_i  (wb_rd_addr_out),

.wb_rd_data_i  (wb_data),

.rs1_data_o    (id_rs1_data),

.rs2_data_o    (id_rs2_data),

.imm_o         (id_imm),

.rs1_addr_o    (id_rs1_addr),

.rs2_addr_o    (id_rs2_addr),

.rd_addr_o     (id_rd_addr),

.alu_op_o      (id_alu_op),

.alu_src_o     (id_alu_src),

.mem_we_o      (id_mem_we),

.mem_re_o      (id_mem_re),

.wb_sel_o      (id_wb_sel),

.reg_we_o      (id_reg_we),

.branch_o      (id_branch),

.jump_o        (id_jump),

.funct3_o      (id_funct3),

.opcode_o      (id_opcode),

.mem_width_o   (id_mem_width),

.mret_o        (id_mret),

.csr_inst_o    (id_csr_inst),

.csr_addr_o    (id_csr_addr),

.csr_op_o      (id_csr_op),

.csr_zimm_o    (id_csr_zimm),

.debug_x0_o    (debug_x0),

.debug_x1_o    (debug_x1),

.debug_x2_o    (debug_x2),

.debug_x3_o    (debug_x3),

.debug_x4_o    (debug_x4),

.debug_x5_o    (debug_x5_t0),

.debug_x6_o    (debug_x6_t1),

.debug_x7_o    (debug_x7_t2),

.debug_x8_o    (debug_x8_t3),

.debug_x9_o    (debug_x9_t4),

.debug_x10_o   (debug_x10_a0),

.debug_x11_o   (debug_x11_a1),

.debug_x12_o   (debug_x12_a2),

.debug_x13_o   (debug_x13),

.debug_x14_o   (debug_x14)

);

id_ex_reg u_id_ex_reg (

.clk_i           (clk_i),

.rst_n_i         (rst_n_i),

.stall_i         (stall_id),

.flush_i         (flush_id),

.intr_flush_i    (intr_flush_ex),

.id_pc_i         (if_id_pc),

.id_rs1_data_i   (id_rs1_data),

.id_rs2_data_i   (id_rs2_data),

.id_imm_i        (id_imm),

.id_rs1_addr_i   (id_rs1_addr),

.id_rs2_addr_i   (id_rs2_addr),

.id_rd_addr_i    (id_rd_addr),

.id_alu_op_i     (id_alu_op),

.id_alu_src_i    (id_alu_src),

.id_mem_we_i     (id_mem_we),

.id_mem_re_i     (id_mem_re),

.id_mem_width_i  (id_mem_width),

.id_wb_sel_i     (id_wb_sel),

.id_reg_we_i     (id_reg_we),

.id_branch_i     (id_branch),

.id_jump_i       (id_jump),

.id_funct3_i     (id_funct3),

.id_opcode_i     (id_opcode),

.id_csr_inst_i   (id_csr_inst),

.id_csr_addr_i   (id_csr_addr),

.id_csr_op_i     (id_csr_op),

.id_csr_zimm_i   (id_csr_zimm),

.ex_pc_o         (id_ex_pc),

.ex_rs1_data_o   (id_ex_rs1_data),

.ex_rs2_data_o   (id_ex_rs2_data),

.ex_imm_o        (id_ex_imm),

.ex_rs1_addr_o   (id_ex_rs1_addr),

.ex_rs2_addr_o   (id_ex_rs2_addr),

.ex_rd_addr_o    (id_ex_rd_addr),

.ex_alu_op_o     (id_ex_alu_op),

.ex_alu_src_o    (id_ex_alu_src),

.ex_mem_we_o     (id_ex_mem_we),

.ex_mem_re_o     (id_ex_mem_re),

.ex_mem_width_o  (id_ex_mem_width),

.ex_wb_sel_o     (id_ex_wb_sel),

.ex_reg_we_o     (id_ex_reg_we),

.ex_branch_o     (id_ex_branch),

.ex_jump_o       (id_ex_jump),

.ex_funct3_o     (id_ex_funct3),

.ex_opcode_o     (id_ex_opcode),

.id_mret_i       (id_mret),

.ex_mret_o       (id_ex_mret),

.ex_csr_inst_o   (id_ex_csr_inst),

.ex_csr_addr_o   (id_ex_csr_addr),

.ex_csr_op_o     (id_ex_csr_op),

.ex_csr_zimm_o   (id_ex_csr_zimm)

);

hazard_unit u_hazard_unit (

.clk_i            (clk_i),

.rst_n_i          (rst_n_i),

.id_rs1_addr_i    (id_rs1_addr),

.id_rs2_addr_i    (id_rs2_addr),

.id_ex_rd_addr_i  (id_ex_rd_addr),

.id_ex_reg_we_i   (id_ex_reg_we),

.id_ex_mem_re_i   (id_ex_mem_re),

.ex_mem_rd_addr_i (ex_mem_rd_addr_for_hazard),

.ex_mem_reg_we_i  (ex_mem_reg_we_for_hazard),

.ex_mem_mem_re_i  (ex_mem_mem_re_for_hazard),

.branch_taken_i   (ex_branch_taken),

.jump_taken_i     (ex_jump_taken),

.interrupt_taken_i(interrupt_taken_pipe),

.interrupt_flush_i(interrupt_flush_pipe),

.stall_if_o       (stall_if),

.stall_id_o       (stall_id),

.flush_if_o       (flush_if),

.flush_id_o       (flush_id),

.intr_flush_if_o  (intr_flush_if),

.intr_flush_id_o  (intr_flush_id),

.intr_flush_ex_o  (intr_flush_ex),

.intr_flush_mem_o (intr_flush_mem),

.intr_flush_wb_o  (intr_flush_wb),

.debug_load_use_hazard_o (hazard_load_use),

.debug_control_hazard_o  (hazard_control),

.debug_id_rs1_addr_o     (hazard_rs1_addr),

.debug_id_rs2_addr_o     (hazard_rs2_addr),

.debug_id_ex_rd_addr_o   (hazard_ex_rd_addr),

.debug_id_ex_reg_we_o    (hazard_ex_reg_we),

.debug_id_ex_mem_re_o    (hazard_ex_mem_re)

);

forwarding_unit u_forwarding_unit (

.id_ex_rs1_addr_i   (id_ex_rs1_addr),

.id_ex_rs2_addr_i   (id_ex_rs2_addr),

.ex_mem_rd_addr_i   (ex_mem_rd_addr),

.ex_mem_reg_we_i    (ex_mem_reg_we),

.ex_mem_mem_re_i    (ex_mem_mem_re),

.mem_wb_rd_addr_i   (wb_rd_addr),

.mem_wb_reg_we_i    (wb_reg_we),

.stall_i            (stall_id),

.forwardA_o         (forwardA),

.forwardB_o         (forwardB),

.debug_ex_mem_rd_addr_o (fwd_ex_mem_rd),

.debug_ex_mem_reg_we_o  (fwd_ex_mem_reg_we),

.debug_mem_wb_rd_addr_o (fwd_mem_wb_rd),

.debug_mem_wb_reg_we_o  (fwd_mem_wb_reg_we)

);

ex_top u_ex_top (

.clk_i              (clk_i),

.rst_n_i            (rst_n_i),

.rs1_data_i         (id_ex_rs1_data),

.rs2_data_i         (id_ex_rs2_data),

.imm_i              (id_ex_imm),

.pc_i               (id_ex_pc),

.wb_sel_i           (id_ex_wb_sel),

.reg_we_i           (id_ex_reg_we),

.rd_addr_i          (id_ex_rd_addr),

.alu_op_i           (id_ex_alu_op),

.alu_src_i          (id_ex_alu_src),

.branch_i           (id_ex_branch),

.jump_i             (id_ex_jump),

.funct3_i           (id_ex_funct3),

.mem_we_i           (id_ex_mem_we),

.mem_re_i           (id_ex_mem_re),

.mem_width_i        (id_ex_mem_width),

.ex_forward_data_i  (ex_mem_alu_result),

.mem_forward_data_i (forward_mem_data),

.forwardA_i         (forwardA),

.forwardB_i         (forwardB),

.opcode_i           (id_ex_opcode),

.csr_result_i       (csr_result),

.alu_result_o       (ex_alu_result),

.mem_addr_o         (ex_mem_addr),

.mem_wdata_o        (ex_mem_wdata),

.mem_we_o           (ex_mem_we),

.mem_re_o           (ex_mem_re),

.branch_taken_o     (ex_branch_taken),

.branch_target_o    (ex_branch_target),

.jump_taken_o       (ex_jump_taken),

.jump_target_o      (ex_jump_target),

.pc_plus4_o         (ex_pc_plus4),

.ex_result_o        (ex_result),

.wb_sel_o           (ex_wb_sel),

.reg_we_o           (ex_reg_we),

.rd_addr_o          (ex_rd_addr),

.mem_width_o        (ex_mem_width),

.op1_selected_o     (op1_selected),

.op2_selected_o     (op2_selected),

.ex_csr_result_o    (ex_csr_result),

.csr_mepc_i         (mepc),

.mret_i             (id_ex_mret)

);

ex_mem_reg u_ex_mem_reg (

.clk_i              (clk_i),

.rst_n_i            (rst_n_i),

.stall_i            (stall_mem),

.flush_i            (flush_mem),

.intr_flush_i       (intr_flush_mem),

.ex_alu_result_i    (ex_alu_result),

.ex_mem_addr_i      (ex_mem_addr),

.ex_mem_wdata_i     (ex_mem_wdata),

.ex_pc_plus4_i      (ex_pc_plus4),

.ex_rd_addr_i       (ex_rd_addr),

.ex_mem_we_i        (ex_mem_we),

.ex_mem_re_i        (ex_mem_re),

.ex_mem_width_i     (ex_mem_width),

.ex_wb_sel_i        (ex_wb_sel),

.ex_reg_we_i        (ex_reg_we),

.ex_csr_result_i    (ex_csr_result),

.mem_alu_result_o   (ex_mem_alu_result),

.mem_mem_addr_o     (ex_mem_mem_addr),

.mem_mem_wdata_o    (ex_mem_mem_wdata),

.mem_pc_plus4_o     (ex_mem_pc_plus4),

.mem_rd_addr_o      (ex_mem_rd_addr),

.mem_mem_we_o       (ex_mem_mem_we),

.mem_mem_re_o       (ex_mem_mem_re),

.mem_mem_width_o    (ex_mem_mem_width),

.mem_wb_sel_o       (ex_mem_wb_sel),

.mem_reg_we_o       (ex_mem_reg_we),

.mem_csr_result_o   (ex_mem_csr_result)

);

mem_top_fpga u_mem_top (

.clk_i             (clk_i),

.rst_n_i           (rst_n_i),

.alu_result_i      (ex_mem_alu_result),

.wdata_i           (ex_mem_mem_wdata),

.mem_we_i          (ex_mem_mem_we),

.mem_re_i          (ex_mem_mem_re),

.mem_width_i       (ex_mem_mem_width),

.pc_plus4_i        (ex_mem_pc_plus4),

.reg_we_i          (ex_mem_reg_we),

.wb_sel_i          (ex_mem_wb_sel),

.rd_addr_i         (ex_mem_rd_addr),

.pc_plus4_o        (mem_pc_plus4),

.reg_we_o          (mem_reg_we),

.wb_sel_o          (mem_wb_sel),

.rd_addr_o         (mem_rd_addr),

.bus_re_o          (mem_bus_re),

.bus_we_o          (mem_bus_we),

.bus_addr_o        (mem_bus_addr),

.bus_wdata_o       (mem_bus_wdata),

.bus_width_o       (mem_bus_width),

.bus_rdata_i       (mem_bus_rdata),

.bus_ready_i       (mem_bus_ready),

.mem_exception_o   ()

);

mem_wb_reg u_mem_wb_reg (

.clk_i            (clk_i),

.rst_n_i          (rst_n_i),

.stall_i          (stall_wb),

.flush_i          (flush_wb),

.intr_flush_i     (intr_flush_wb),

.mem_alu_result_i (ex_mem_alu_result),

.mem_mem_rdata_i  (mem_bus_rdata),

.mem_pc_plus4_i   (mem_pc_plus4),

.mem_rd_addr_i    (mem_rd_addr),

.mem_wb_sel_i     (mem_wb_sel),

.mem_reg_we_i     (mem_reg_we),

.mem_csr_result_i (ex_mem_csr_result),

.mem_mem_re_i     (ex_mem_mem_re),

.wb_mem_re_o      (mem_wb_mem_re),

.wb_alu_result_o  (wb_alu_result),

.wb_mem_rdata_o   (wb_mem_rdata),

.wb_pc_plus4_o    (wb_pc_plus4),

.wb_rd_addr_o     (wb_rd_addr),

.wb_wb_sel_o      (wb_wb_sel),

.wb_reg_we_o      (wb_reg_we),

.wb_csr_result_o  (wb_csr_result)

);

wb_top u_wb_top (

.clk_i          (clk_i),

.rst_n_i        (rst_n_i),

.alu_result_i   (wb_alu_result),

.mem_rdata_i    (wb_mem_rdata),

.pc_plus4_i     (wb_pc_plus4),

.csr_result_i   (wb_csr_result),

.wb_sel_i       (wb_wb_sel),

.reg_we_i       (wb_reg_we),

.rd_addr_i      (wb_rd_addr),

.wb_data_o      (wb_data),

.reg_we_o       (wb_reg_we_out),

.rd_addr_o      (wb_rd_addr_out)

);

bus_arbiter u_bus_arbiter (

.clk_i          (clk_i),

.rst_n_i        (rst_n_i),

.mem_re_i       (mem_bus_re),

.mem_we_i       (mem_bus_we),

.mem_addr_i     (mem_bus_addr),

.mem_wdata_i    (mem_bus_wdata),

.mem_width_i    (mem_bus_width),

.mem_rdata_o    (mem_bus_rdata),

.mem_ready_o    (mem_bus_ready),

.ram_re_o       (bus_ram_re),

.ram_we_o       (bus_ram_we),

.ram_addr_o     (bus_ram_addr),

.ram_wdata_o    (bus_ram_wdata),

.ram_width_o    (bus_ram_width),

.ram_rdata_i    (bus_ram_rdata),

.ram_ready_i    (bus_ram_ready),

.uart_we_o      (bus_uart_we),

.uart_re_o      (bus_uart_re),

.uart_addr_o    (bus_uart_addr),

.uart_wdata_o   (bus_uart_wdata),

.uart_rdata_i   (bus_uart_rdata),

.gpio_we_o      (bus_gpio_we),

.gpio_re_o      (bus_gpio_re),

.gpio_addr_o    (bus_gpio_addr),

.gpio_wdata_o   (bus_gpio_wdata),

.gpio_rdata_i   (bus_gpio_rdata),

.timer_we_o     (bus_timer_we),

.timer_re_o     (bus_timer_re),

.timer_addr_o   (bus_timer_addr),

.timer_wdata_o  (bus_timer_wdata),

.timer_rdata_i  (bus_timer_rdata)

);

uart_ctrl #(

.CLK_FREQ(200_000_000),

.BAUD_RATE(115200)

) u_uart_ctrl (

.clk_i                  (clk_i),

.rst_n_i                (rst_n_i),

.we_i                   (bus_uart_we),

.addr_i                 (bus_uart_addr),

.wdata_i                (bus_uart_wdata),

.rdata_o                (bus_uart_rdata),

.tx_pin_o               (uart_tx_o),

.tx_data_o              (tx_data),

.tx_valid_o             (tx_valid),

.debug_fifo_out_data_o  (debug_fifo_out_data_o),

.direct_transfer_o      (direct_transfer_o),

.data_to_send_o         (data_to_send_o),

.tx_ready_o             (tx_ready),

.debug_fifo_we_reg_o    (debug_fifo_we_reg),

.debug_state_o          (debug_uart_state),

.debug_baud_cnt_o       (debug_uart_baud_cnt),

.debug_bit_cnt_o        (debug_uart_bit_cnt),

.debug_shift_reg_o      (debug_uart_shift_reg),

.debug_fifo_data0_o     (debug_uart_fifo_data0),

.debug_fifo_data1_o     (debug_uart_fifo_data1),

.debug_fifo_data2_o     (debug_uart_fifo_data2),

.debug_fifo_data3_o     (debug_uart_fifo_data3),

.debug_fifo_data4_o     (debug_uart_fifo_data4),

.debug_fifo_data5_o     (debug_uart_fifo_data5),

.debug_fifo_data6_o     (debug_uart_fifo_data6),

.debug_fifo_data7_o     (debug_uart_fifo_data7),

.debug_fifo_data8_o     (debug_uart_fifo_data8),

.debug_fifo_data9_o     (debug_uart_fifo_data9),

.debug_fifo_data10_o    (debug_uart_fifo_data10),

.debug_fifo_data11_o    (debug_uart_fifo_data11),

.debug_fifo_data12_o    (debug_uart_fifo_data12),

.debug_fifo_data13_o    (debug_uart_fifo_data13),

.debug_fifo_data14_o    (debug_uart_fifo_data14),

.debug_fifo_data15_o    (debug_uart_fifo_data15),

.debug_wr_ptr_o         (debug_uart_wr_ptr),

.debug_rd_ptr_o         (debug_uart_rd_ptr),

.debug_fifo_count_o     (debug_uart_fifo_count),

.debug_fifo_full_o      (debug_uart_fifo_full),

.debug_fifo_empty_o     (debug_uart_fifo_empty),

.debug_fifo_we_o        (debug_uart_fifo_we),

.debug_fifo_re_o        (debug_uart_fifo_re)

);

gpio u_gpio (

.clk_i        (clk_i),

.rst_n_i      (rst_n_i),

.we_i         (bus_gpio_we),

.re_i         (bus_gpio_re),

.addr_i       (bus_gpio_addr),

.wdata_i      (bus_gpio_wdata),

.rdata_o      (bus_gpio_rdata),

.gpio_in_i    (gpio_in_test),

.gpio_out_o   (gpio_out_test),

.gpio_oe_o    (gpio_oe_test),

.interrupt_o  (gpio_interrupt),

.debug_gpio_out  (gpio_out_val),

.debug_gpio_oe   (gpio_oe_val),

.debug_gpio_in   (gpio_in_val),

.debug_gpio_if   (gpio_if_val)

);

timer u_timer (

.clk_i        (clk_i),

.rst_n_i      (rst_n_i),

.we_i         (bus_timer_we),

.re_i         (bus_timer_re),

.addr_i       (bus_timer_addr),

.wdata_i      (bus_timer_wdata),

.rdata_o      (bus_timer_rdata),

.interrupt_o  (timer_interrupt),

.debug_load_value (timer_load_val),

.debug_counter    (timer_count_val),

.debug_enable     (timer_enable),

.debug_irq_flag   (timer_irq_flag)

);

assign intr_timer = timer_interrupt;

assign debug_timer_irq = timer_interrupt;

assign debug_timer_counter = 7'b0;

csr_regfile u_csr_regfile (

.clk_i              (clk_i),

.rst_n_i            (rst_n_i),

.csr_addr_i         (id_ex_csr_addr),

.csr_rdata_o        (csr_rdata),

// 独立写端口

.csr_inst_we_i      (csr_inst_we),

.csr_inst_waddr_i   (csr_inst_waddr),

.csr_inst_wdata_i   (csr_inst_wdata),

.csr_mepc_we_i      (pipe_csr_mepc_we),

.csr_mepc_data_i    (pipe_csr_mepc_data),

.csr_mcause_we_i    (pipe_csr_mcause_we),

.csr_mcause_data_i  (pipe_csr_mcause_data),

.csr_mstatus_we_i   (pipe_csr_mstatus_we),

.csr_mstatus_data_i (pipe_csr_mstatus_data),

.intr_software_i    (intr_software),

.intr_timer_i       (intr_timer),

.intr_external_i    (intr_external),

.mtvec_o            (mtvec),

.mepc_o             (mepc),

.mcause_o           (mcause),

.mie_o              (mie),

.mstatus_o          (mstatus),

.mip_o              (mip),

.debug_mstatus_o    (debug_mstatus),

.debug_mie_o        (debug_mie),

.debug_mtvec_o      (debug_mtvec),

.debug_mepc_o       (debug_mepc),

.debug_mcause_o     (debug_mcause)

);

assign csr_inst_valid = id_ex_csr_inst;

assign csr_op         = id_ex_csr_op;

assign csr_addr       = id_ex_csr_addr;

assign csr_rs1_addr   = id_ex_rs1_addr;

assign csr_rs1_data   = op1_selected;

assign csr_imm        = id_ex_imm;

assign debug_mip      = mip;

csr_instructions u_csr_instructions (

.clk_i              (clk_i),

.rst_n_i            (rst_n_i),

.csr_inst_valid_i   (csr_inst_valid),

.csr_op_i           (csr_op),

.csr_addr_i         (csr_addr),

.rs1_addr_i         (csr_rs1_addr),

.rs1_data_i         (csr_rs1_data),

.imm_i              (csr_imm),

.csr_rdata_i        (csr_rdata),

.csr_we_o           (csr_inst_we),

.csr_waddr_o        (csr_inst_waddr),

.csr_wdata_o        (csr_inst_wdata),

.csr_result_o       (csr_inst_result),

.debug_csr_op_type_o(),

.debug_csr_we_o     (),

.debug_csr_addr_o   ()

);

interrupt_controller u_interrupt_controller (

.clk_i              (clk_i),

.rst_n_i            (rst_n_i),

.intr_software_i    (intr_software),

.intr_timer_i       (intr_timer),

.intr_external_i    (intr_external),

.mie_i              (mie),

.mip_i              (mip),

.mstatus_i          (mstatus),

.mtvec_i            (mtvec),

.intr_pending_o     (intr_pending),

.intr_cause_o       (intr_cause),

.intr_handler_addr_o(intr_handler_addr)

);

interrupt_pipeline u_interrupt_pipeline (

.clk_i              (clk_i),

.rst_n_i            (rst_n_i),

.if_pc_i            (if_pc),

.id_valid_i         (|if_id_instr),

.id_pc_i            (if_id_pc),

.ex_valid_i         (|id_ex_opcode),

.ex_pc_i            (id_ex_pc),

.ex_branch_taken_i  (ex_branch_taken),

.ex_jump_taken_i    (ex_jump_taken),

.mem_valid_i        (ex_mem_mem_we || ex_mem_mem_re),

.mem_pc_i           (ex_mem_pc_plus4 - 4),

.mem_mem_re_i       (ex_mem_mem_re),

.mem_mem_we_i       (ex_mem_mem_we),

.wb_valid_i         (wb_reg_we_out),

.wb_rd_addr_i       (wb_rd_addr_out),

.wb_reg_we_i        (wb_reg_we_out),

.intr_pending_i     (intr_pending),

.intr_cause_i       (intr_cause),

.mstatus_i          (mstatus),

.csr_mepc_we_o      (pipe_csr_mepc_we),

.csr_mepc_data_o    (pipe_csr_mepc_data),

.csr_mcause_we_o    (pipe_csr_mcause_we),

.csr_mcause_data_o  (pipe_csr_mcause_data),

.csr_mstatus_we_o   (pipe_csr_mstatus_we),

.csr_mstatus_data_o (pipe_csr_mstatus_data),

.interrupt_taken_o  (interrupt_taken_pipe),

.interrupt_flush_o  (interrupt_flush_pipe),

.interrupt_pc_o     (interrupt_pc_pipe),

.debug_interrupt_accepted     (debug_interrupt_accepted),

.debug_interrupt_hold_cnt     (debug_interrupt_hold_cnt),

.debug_interrupt_condition    (debug_interrupt_condition),

.debug_interrupt_condition_bits(debug_interrupt_condition_bits),

.debug_selected_pc            (debug_selected_pc),

.debug_selected_stage         (debug_selected_stage)

);

// ==========================================================================

// CSR写信号合并

// ==========================================================================

wire [31:0] final_csr_wdata;

wire [11:0] final_csr_waddr;

wire        final_csr_we;

// assign final_csr_we    = csr_inst_we ||

//                          pipe_csr_mepc_we || pipe_csr_mcause_we || pipe_csr_mstatus_we;

// assign final_csr_waddr = pipe_csr_mepc_we ? CSR_MEPC :

//                          pipe_csr_mcause_we ? CSR_MCAUSE :

//                          pipe_csr_mstatus_we ? CSR_MSTATUS :

//                          csr_inst_waddr;

// assign final_csr_wdata = pipe_csr_mepc_we ? pipe_csr_mepc_data :

//                          pipe_csr_mcause_we ? pipe_csr_mcause_data :

//                          pipe_csr_mstatus_we ? pipe_csr_mstatus_data :

//                          csr_inst_wdata;

assign debug_interrupt_taken_pipe = interrupt_taken_pipe;

// assign csr_we    = final_csr_we;

// assign csr_waddr = final_csr_waddr;

// assign csr_wdata = final_csr_wdata;

assign csr_result = csr_inst_result;

assign debug_ex_csr_result = ex_csr_result;

assign debug_ex_mem_csr_result = ex_mem_csr_result;

// ==========================================================================

// 调试信号连接

// ==========================================================================

assign debug_if_pc          = if_pc;

assign debug_if_instr       = if_instr;

assign debug_id_rs1_data    = id_rs1_data;

assign debug_id_rs2_data    = id_rs2_data;

assign debug_id_rs1_addr    = id_rs1_addr;

assign debug_id_rs2_addr    = id_rs2_addr;

assign debug_ex_alu_result  = ex_alu_result;

assign debug_ex_mem_addr    = ex_mem_addr;

assign debug_ex_mem_wdata   = ex_mem_wdata;

assign debug_ex_mem_we      = ex_mem_we;

assign debug_ex_mem_width   = ex_mem_width;

assign debug_mem_bus_we     = mem_bus_we;

assign debug_mem_bus_addr   = mem_bus_addr;

assign debug_mem_bus_wdata  = mem_bus_wdata;

assign debug_mem_bus_rdata  = mem_bus_rdata;

assign debug_bus_uart_we    = bus_uart_we;

assign debug_bus_uart_addr  = bus_uart_addr;

assign debug_bus_uart_wdata = bus_uart_wdata;

assign debug_interrupt_pending = intr_pending;

assign debug_interrupt_taken   = interrupt_taken_pipe;

assign debug_csr_inst_pc = id_ex_pc;

assign debug_csr_inst_instr = {id_ex_opcode, 25'b0};

assign debug_csr_write = csr_we;

assign debug_csr_write_addr = csr_waddr;

assign debug_csr_write_data = csr_wdata;

assign debug_t0_value = id_rs1_data;

assign debug_instr_4 = (if_pc == 32'h10) ? if_instr : 32'b0;

assign debug_instr_5 = (if_pc == 32'h14) ? if_instr : 32'b0;

assign debug_instr_6 = (if_pc == 32'h18) ? if_instr : 32'b0;

assign debug_instr_7 = (if_pc == 32'h1C) ? if_instr : 32'b0;

assign debug_id_csr_inst = id_csr_inst;

assign debug_id_csr_addr = id_csr_addr;

assign debug_id_csr_op = id_csr_op;

assign debug_id_csr_zimm = id_csr_zimm;

assign debug_ex_csr_inst = id_ex_csr_inst;

assign debug_ex_csr_addr = id_ex_csr_addr;

assign debug_ex_csr_op = id_ex_csr_op;

assign debug_ex_csr_zimm = id_ex_csr_zimm;

assign debug_id_ex_flush = flush_id;

assign debug_id_ex_intr_flush = intr_flush_ex;

assign debug_id_ex_stall = stall_id;

assign ex_mem_rd_addr_for_hazard = ex_mem_rd_addr;

assign ex_mem_reg_we_for_hazard = ex_mem_reg_we;

assign ex_mem_mem_re_for_hazard = ex_mem_mem_re;

assign forward_mem_data = mem_wb_mem_re ? wb_mem_rdata : wb_alu_result;

assign debug_csr_inst_valid = csr_inst_valid;

assign debug_csr_inst_op = csr_op;

assign debug_csr_inst_addr = csr_addr;

assign debug_csr_inst_rs1 = csr_rs1_addr;

assign debug_csr_inst_rs1_data = csr_rs1_data;

assign debug_csr_inst_imm = csr_imm;

assign debug_csr_inst_rdata = csr_rdata;

assign debug_csr_inst_we = csr_inst_we;

assign debug_csr_inst_waddr = csr_inst_waddr;

assign debug_csr_inst_wdata = csr_inst_wdata;

assign debug_csr_inst_result = csr_inst_result;

assign debug_final_csr_we = final_csr_we;

assign debug_final_csr_waddr = final_csr_waddr;

assign debug_final_csr_wdata = final_csr_wdata;

assign debug_csr_reg_rdata = csr_rdata;

assign debug_csr_reg_we = csr_we;

assign debug_csr_reg_waddr = csr_waddr;

assign debug_csr_reg_wdata = csr_wdata;

assign debug_forwardA = forwardA;

assign debug_forwardB = forwardB;

assign debug_id_ex_rs1 = id_ex_rs1_addr;

assign debug_id_ex_rs2 = id_ex_rs2_addr;

assign debug_ex_mem_rd = ex_mem_rd_addr;

assign debug_ex_mem_reg_we = ex_mem_reg_we;

assign debug_ex_mem_mem_re = ex_mem_mem_re;

assign debug_mem_wb_rd = wb_rd_addr;

assign debug_mem_wb_reg_we = wb_reg_we;

assign debug_ex_mem_alu_result = ex_mem_alu_result;

assign debug_mem_forward_data = wb_alu_result;

assign debug_op1_selected = op1_selected;

assign debug_op2_selected = op2_selected;

assign debug_rs1_addr_id = id_rs1_addr;

assign debug_rs2_addr_id = id_rs2_addr;

assign debug_rs1_data_id = id_rs1_data;

assign debug_rs2_data_id = id_rs2_data;

assign debug_stall_if         = stall_if;

assign debug_stall_id         = stall_id;

assign debug_flush_if         = flush_if;

assign debug_flush_id         = flush_id;

assign debug_load_use_hazard  = hazard_load_use;

assign debug_control_hazard   = hazard_control;

assign debug_hazard_rs1_addr  = hazard_rs1_addr;

assign debug_hazard_rs2_addr  = hazard_rs2_addr;

assign debug_hazard_ex_rd_addr = hazard_ex_rd_addr;

assign debug_hazard_ex_reg_we = hazard_ex_reg_we;

assign debug_hazard_ex_mem_re = hazard_ex_mem_re;

assign debug_fwd_ex_mem_rd    = fwd_ex_mem_rd;

assign debug_fwd_ex_mem_reg_we = fwd_ex_mem_reg_we;

assign debug_fwd_mem_wb_rd    = fwd_mem_wb_rd;

assign debug_fwd_mem_wb_reg_we = fwd_mem_wb_reg_we;

assign debug_ex_rs1_original  = id_ex_rs1_data;

assign debug_ex_rs2_original  = id_ex_rs2_data;

assign debug_next_pc          = (ex_branch_taken ? ex_branch_target :

ex_jump_taken ? ex_jump_target :

(stall_if ? if_pc : if_pc + 4));

assign debug_pc_changed       = (if_pc != debug_next_pc);

assign debug_gpio_out      = gpio_out_val;

assign debug_gpio_oe       = gpio_oe_val;

assign debug_gpio_in       = gpio_in_val;

assign debug_gpio_if       = gpio_if_val;

assign debug_gpio_interrupt = gpio_interrupt;

assign debug_bus_gpio_we   = bus_gpio_we;

assign debug_bus_gpio_re   = bus_gpio_re;

assign debug_bus_gpio_addr = bus_gpio_addr;

assign debug_bus_gpio_wdata = bus_gpio_wdata;

assign debug_bus_gpio_rdata = bus_gpio_rdata;

assign debug_timer_load     = timer_load_val;

assign debug_timer_count    = timer_count_val;

assign debug_timer_enable   = timer_enable;

assign debug_timer_irq_flag = timer_irq_flag;

assign debug_timer_interrupt = timer_interrupt;

// ==========================================================================

// 仿真监控

// ==========================================================================

integer cycle_counter = 0;

reg [31:0] last_pc = 0;

always @(posedge clk_i) begin

if (rst_n_i) begin

cycle_counter <= cycle_counter + 1;

if (if_pc !== last_pc) begin

last_pc <= if_pc;

if (cycle_counter < 500) begin

$display("[%0t] Cycle %0d: PC=%h, Instr=%h",

$time, cycle_counter, if_pc, if_instr);

end

end

if (intr_pending) begin

$display("[%0t] Cycle %0d: *** INTERRUPT PENDING *** cause=%h",

$time, cycle_counter, intr_cause);

end

if (interrupt_taken_pipe) begin

$display("[%0t] Cycle %0d: *** INTERRUPT TAKEN *** handler=%h mepc=%h",

$time, cycle_counter, intr_handler_addr, mepc);

end

if (cycle_counter == 10) begin

$display("[%0t] CSR Status: mtvec=%h, mie=%h, mstatus=%h",

$time, mtvec, mie, mstatus);

end

if (csr_we) begin

$display("[%0t] Cycle %0d: CSR Write: addr=%h data=%h",

$time, cycle_counter, csr_waddr, csr_wdata);

end

if (bus_uart_we) begin

$display("[%0t] Cycle %0d: UART WRITE: Addr=%h, Data=%h ('%c')",

$time, cycle_counter, bus_uart_addr, bus_uart_wdata,

bus_uart_wdata[7:0]);

end

if (stall_if || stall_id || flush_if || flush_id ||

intr_flush_if || intr_flush_id) begin

$display("[%0t] Cycle %0d: Pipeline Control: stall_if=%b, stall_id=%b, flush_if=%b, flush_id=%b, intr_flush_if=%b",

$time, cycle_counter, stall_if, stall_id, flush_if, flush_id, intr_flush_if);

end

if (if_instr == 32'h30200073) begin

$display("[%0t] Cycle %0d: *** MRET INSTRUCTION ***",

$time, cycle_counter);

end

if (bus_uart_we && bus_uart_wdata[7:0] == 8'h2A) begin

$display("[%0t] Cycle %0d: *** INTERRUPT HANDLER EXECUTED: sent '*' ***",

$time, cycle_counter);

end

end else begin

cycle_counter <= 0;

last_pc <= 32'b0;

end

end

endmodule